开始刷新BIOS前请注意几点 Before reflash the BOIS ,pls noted as below

- 保证电池电量充足，尽量插上电源适配器刷新   make suer the battery have enough power, and best with adapter

- 保证所用OTG线缆的完好性，以免刷BIOS途中U盘断开  make suer the OTG cable is ok for use and avoid U key will out of connect during reflash the BIOS

Shell环境下如何刷BIOS，详细步骤如下所示： how to reflash the BIOS under the shell,pls follow below step

- 插上U盘，开机，按F7键进入引导界面，选择UEFI:Built-in EFI Shell按Enter回车键进入Shell环境，如下图：  Put the U key frist ,then turn on the tablet and press F7 into the guide,choose UEFI:Built-in EFI Shell and press Enter into shell,as below

- 进入Shell环境后，确定U盘对应的映射区间。键入Map –b命令，查看分区映射关系，找到Removable HardDisk选项对应的映射区间。如下图是fs3:，然后键入fs3:(注意后面有：)，如下图所示  when into shell ,make suer the U key is match the disk. Type Map –b to order,and check the disk relation,find Removable HardDisk to choosed the correct disk.as below photo showing is  fs3:,then type fs3:(pls noted need with :),as blow photo showing

- 键入fs3:  进入U盘后，使用Dir命令查看根目录，找到BIOS文件夹，如下图BIOS文件夹是bb，然后cd bb命令进入bb文件夹路径，使用Dir命令查看根目录，最后输入32位BIOS命令：Fpt32.efi  –f   i8270xxxxxxxxxxx.bin(BIOS文件). Type fs3: into U key ,use Dir order to check the list, find BIOS file,as below showing BIOS file name is bb,then type cd bb to order into bb file ,use Dir order to check the file ,and type 32bit BIOS order: Fpt32.efi  –f   i8270xxxxxxxxxxx.bin(this for BIOS file name).

64位BIOS命令：Fpt64.efi  –f   i8270xxxxxxxxxxx.bin(BIOS文件). For 64 bit BIOS order is : Fpt64.efi  –f   i8270xxxxxxxxxxx.bin(this for BIOS file name)